D032

CENTRE LOGO HERE

Test Centre Name

Test Centre Street

Centre Town, Centre County

Postcode:                   Tel:

##

CYLINDER INSPECTION & TEST WORKSHEET           JOB No:  ____________

| CYL Make: | _________ | Serial No: | ___________________ |
| --- | --- | --- | --- |
| Spec: ______________ Matl: Steel ☐ Alloy ☐ Composite ☐ |  |  |  |
| Capacity: ___ Litre WP ______ bar/psi TP ______ bar/psi Manufactured Date: _________ |  |  |  |
| RISK ASSESSMENT Cylinder Use: Recreational ☐ Offshore ☐ Inshore ☐ Media ☐ Scientific ☐ Police ☐ MoD ☐ Risk of Water Ingress: Y ☐ N ☐ Next Inspection: 6 mths. ☐ 2½ Yrs. ☐ 5 Yrs ☐ Other: _______ This Inspection: PIAT (Hydro) ☐ PI (Visual) ☐ CYLINDER EMPTY: Y ☐ N ☐ Valve Blocked: Y ☐ N ☐ |  |  |  |
| VALVE Make: ________________ Serial No: ________________ WP: ______ |  |  |  |
| EXTERNAL CONDITION GOOD ☐ Satisfactory ☐ Remedial action required: Y ☐ N ☐ Action: _____________________ |  |  |  |
| WALL THICKNESS Manufacturer’s min Wall _______mm Measured wall min ______ m Pass ☐ Fail ☐ Manufacturer’s min Base _______mm Measured base min ______ mm Pass ☐ Fail ☐ |  |  |  |
| CYLINDER NECK THREADS Cylinder Threads: 25mm ☐ G ¾ ☐ 18mm ☐ 0.715 ☐ 0.6 ☐ 25E ☐ Other: ________ Neck Threads: SATISFACTORY ☐ FAIL ☐ Not-Go Turns ______ Go Full Turns: Y ☐ N ☐ DEFECTS FOUND ______________________ EDDY CURRENT TEST: N/A ☐ Pass ☐ Fail ☐ |  |  |  |
| VALVE INSPECTION & SERVICE Stem Threads: 25mm ☐ G ¾ ☐ 18mm ☐ 0.715 ☐ 0.6 ☐ 25E ☐ Other: ________ Stem Threads: Satisfactory ☐ Fail ☐ Not-Go Turns _____ Outlet Threads: G 5/8 ☐ M26 ☐ Outlet Threads: Satisfactory ☐ Fail ☐ Not-Go Turns _____ DEFECTS____________________ VALVE SERVICED ☐ VALVE RENEWAL ☐ VALVE AND CYLINDER THREADS MATCH: Y ☐ N ☐ |  |  |  |
| INTERNAL CONDITION GOOD ☐ SATISFACTORY ☐ SHOT BLAST REQUIRED: Y ☐ N ☐ Pass ☐ Fail ☐ DEFECTS FOUND ______________________________ NDT Crack Detection (Compulsory for AA6351) N/A ☐ Conducted: Y ☐ N ☐ Pass ☐ Fail ☐ |  |  |  |
| TARE WEIGHT (Supplementary test, only required if there is doubt after other tests) Stamped Tare (T) ____Kg, Actual Tare (A) _____ Kg, % Weight Loss: T-ATx100 = _____% Pass ☐ Fail ☐ |  |  |  |
| PRESSURE TEST ☐ Visual Only ☐ Proof Test Y ☐ N ☐ Confirmed Dry before test ☐, Confirmed Dry after test ☐. Pass ☐ Fail ☐ Volumetric Test: Y ☐ N ☐ Initial Reading (I) ______ Test Expansion (E) ________ Pressure Released (R) ______ Permanent Set R-IE*100= ______ % Less than Stated % (or 5%) Pass ☐ Fail ☐ |  |  |  |
| QUALITY CHECK Valve Destroyed Y ☐ N ☐, Cylinder Destroyed Y ☐ N ☐, Destruction Record Complete ☐ Dry ☐, New O-Ring ☐, PTFE Tape ☐, Valve Torque _____Nm, Stamp ☐, Resin Label (Composite) ☐ Filled & Leak Test ☐ , IDEST Quadrant: ☐, Completion Tag ☐, Booking Form ☐, Test Cert ☐ |  |  |  |
| Cylinder Inspected in accordance with ISO 18119 ☐, ISO 11623 ☐ Valve inspected and serviced in accordance with ISO 22434 ☐ This cylinder has PASSED ☐ FAILED ☐ FAILED REASON: ____________________________ Signed _______________________ Print ___________________ Date: Click or tap to enter a date. |  |  |  |

Original 	Amended: 07/07/2026